█ Industrial Waste   █ Prescribed Waste   █ Solid Radioactive Waste

█ Recyclable Waste   █ Non-Recyclable Waste   █ Clinical Waste   █ Related Waste   █ Other Prescribed Waste   █ Liquid Waste

|  |  | Waste Type | Description of waste type and examples of waste generated | On-site Management | Bin colours Australian Standard AS 4123.7-2006 Mobile waste containers part 7: colours, markings and designation requirements | Off-site disposal method |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Cardboard boxes (plain and waxed) | Packaging container composed of cardboard materials Examples: Plain cardboard boxes, waxed cardboard boxes, cardboard inserts | Flatten and dispose until: waste disposal container cardboard compactor cardboard storage area | Body: Dark green or black Lid: Blue | Paper and cardboard recycling facility. Cardboard is pulped, cleaned, de-inked, drained and dried before being remade in cardboard. |
|  |  | Commingled recyclables bottles, cans, cartons, paper (full) | Materials that are capable of being remanufactured or reused Examples: Glass bottles, milk cartons, aluminium cans, steel cans, paper, cardboard, newspaper, uncontaminated paper towels, rigid plastics (coded 1–7) | Dispose in: commingled recycling container | Body: Dark green or black Lid: Yellow Please note: The department and SV do not support using the yellow lid in healthcare settings | Materials recycling facility (MRF). Materials are sorted via various separating processes including magnetic steel removal, screening and air separation and prepared for recycling at a processing plant. |
|  |  | E-waste | E-waste consists of obsolete computer, electronic or hardware waste Examples: Computer parts, printers, photocopiers, mobile phones, pagers, monitors, televisions, electronic equipment, discarded electronic medical equipment | Store until collection in: skip cage | Body: Dark green or black Lid: White | E-waste recyclers can dismantle electronic equipment and recycle all the parts, such as the hard plastic covering, copper in wires and steel components. |
|  |  | Note: · Delete or copy all software and information from computer before donating or recycling · CDs and disks can be placed in general waste container for disposal |  |  |  |  |
|  |  | Food/green organic waste | Food scraps, garden clippings and natural fibrous materials that are biodegradable and capable of being composted or put in worm farms Examples: Flowers, grass clippings, leftover food, vegetables scraps, grounds gardening maintenance | Dispose in: green organics container compost bin worm farm | Body: Dark green or black Lid: Lime green | Ideally, food/green organics are taken to a composting facility (if reasonable to do so) |
|  |  | Paper | Examples: Newspaper, wrapping paper, cards, paper cups, envelopes (including window envelopes), office paper, non-confidential files | Dispose in: paper recycling bin | Body: Dark green or black Lid: Blue | Paper and cardboard recycling facility. |
|  |  | Note: · Paper can also be added to your co mingled recycling container – please check with your contractor |  |  |  |  |
|  |  | Confidential paper | Paper documents that contain private, contractual, confidential or information identifying a person Examples: Patient records, medical records file, contracts, staff files | Dispose in: confidential paper recycling bin (locked) | Body: Blue Lid: Blue | Confidential paper is taken to paper and cardboard recycling facility for secure shredding and secure pulping before being drained and dried before being remade into paper |
|  |  | Plastics (other) | Plastics that can not be put in commingled recycling bin Examples: Shrink wrap, plastic wrap, bubble wrap, polystyrene cups and boxes, expanded polystyrene and plastic bags | Store until collection in: skip cage |  | Plastics will be granulated and recycled back into plastics |
|  |  | Photographic waste | Photographic waste generating from X-ray developing equipment or Xray film Examples: Fixer, developer and x-ray films | Store until collection in: bag drum tank |  | Recycled at facility licensed by EPA |
|  |  | Toner/printer cartridges | Used, empty toner, printer cartridges from any type of printer, fax and photocopier Examples: Inkjet and laser printer Cartridges, Toner bottles, drum kits, fuser kits | Store until collection in: printer cartridge recycling box (free service through some contractors and Close the Loop) |  | Cartridge reuse facility. Toner or printer cartridges are refilled for re-use. Cartridge recycling facility. Toner and printers are granulated for recycling. |
|  |  | Batteries | Empty batteries Examples: Use in hearing aids, pagers, digital cameras, communication devices | Store in a non leaking receptacle |  | Metals and chemicals can be recovered for recycling |
|  |  | Construction solid inert/ demolition | Materials generated from building demolition, construction and refurbishment sites Examples: Concrete, wood, flooring, metals, carpet, bricks | Dispose in: recyclables skip general waste skip |  |  |
|  |  | Note: · All construction waste should be separated and recycled where possible |  |  |  |  |
|  |  | Note: · The department advises that a donations disclaimer should assist all donations to avoid legal implications · In the case of X-ray equipment, please notify the department, radiation safety section prior to disposal; the X-ray apparatus must be made inoperable prior to disposal |  |  |  |  |
|  |  | Rubbish/general waste | Waste that is not capable of being composted, recycled, reprocessed or re-used Examples: Incontinence pads, disposable nappies, drained dialysis waste, plastic bags, masks, gloves | Dispose in: general waste bin garbage compactor | Body: Dark green or black Lid: Red | EPA licensed landfill |
|  |  | Sanitary waste | Sanitary waste generating in non-patient areas only Examples: Sanitary waste in ladies public and staff toilets | Dispose in: sanitary waste bin |  | EPA licensed landfill |

|  |  | Waste Type | Description of waste type and examples of waste generated | On-site Management | Bin colours Australian Standard AS 4123.7-2006 Mobile waste containers part 7: colours, markings and designation requirements | Off-site disposal method |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Clinical waste | For clinical waste definition please refer to Waste Management Policy Examples: Human tissue, gloves or bandages containing blood, cultures and specimens,disposable theatre wraps, swabs and soiled dressings | Dispose in: clinical waste container, bucket or bags | Body: Yellow Lid: Yellow Bag: Yellow Symbol: Black biological hazard Marked: Clinical waste | Disposal at a licensed premises |
|  |  | Notes: · Packaging, instruments, trays, and unused materials must be disposed of as general waste unless contaminated. |  |  |  |  |
|  |  | Cytotoxic waste | All equipment and materials that may be contaminated with a cytotoxic drug during preparation, transport or administration of chemotherapy Examples: Disposable gloves and masks, dressing material, IV solution bags, absorbent pads | Dispose in: cytotoxic waste container | Body: Purple, rigid container Lid: Purple Bag: Purple Symbol: Cell undergoing telophase in white Marked: Cytotoxic waste | Waste is collected by an EPA-licensed contractor for incineration at an EPA licensed facility |
|  |  | Cytotoxic waste (sharps) | All equipment and materials that may be contaminated with cytotoxic drug and may pierce a plastic bag Examples: Syringes, needles, drug vials, ampoules. | Dispose in: cytotoxic waste container | Body: Purple ridged container Lid: Purple Symbol: Cell undergoing telophase in White Marked: Cytotoxic waste | Waste is collected by an EPA-licensed contractor for incineration at an EPA licensed facility |
|  |  | Pharmaceutical waste | Any expired or discarded pharmaceutical substance. Also includes filters or materials contaminated with pharmaceutical products. Examples: Expired pharmaceuticals, filters or materials that may be contaminated by pharmaceutical product, returned or no longer required pharmaceuticals | Dispose in: collection boxes marked ‘Pharmaceutical waste, for incineration only’ |  | Waste is collected by an EPA-licensed contractor for incineration at an EPA licensed facility |

|  |  | Waste Type | Description of waste type and examples of waste generated | On-site Management | Off-site disposal method |
| --- | --- | --- | --- | --- | --- |
|  |  | Chemical waste | Chemical waste including waste generated from the use of chemicals in medical applications, domestic services, maintenance, laboratories, sterilisation processes and research processes Examples: Mercury, cyanide, formalin, glutaraldehyde, photochemical waste, toluene and xylene | Controlled release to sewer in accordance with trade waste agreement with water authority | Search the Prescribed Industrial Waste Database for transporters & treaters |